The following equipment is mandatory at all 24H SERIES races.
All order forms can be found on the 24H SERIES website: www.24hseries.com/documents

- MyLaps X2 Racelink (Club or Pro)
Timeservice is the official reseller of the MyLaps X2 RaceLink. The Club version is sufficient for the 24H SERIES events. The X2 RaceLink enables us to show the live GPS position of all cars and this needs to be connected to the LUMIRANK LED Position display and Driver Information Display.

- Transponder including driver-ID
Timeservice is the official reseller of the MyLaps Transponders including driver-ID.

- LUMIRANK, Driver Information Display & RGB Light
The LUMIRANK, DID and RGB are provided by Creventic and the rental is included in the entry fee for the race. A deposit of 1.500 EUR is required. 

LUMIRANK LED POSITION DISPLAY
The LUMIRANK LED Position Display on the front screen shows the position of the car in its class and the 3-letter code of the driver in the car.

STS DRIVER INFORMATION DISPLAY
The Driver Information Display needs to be placed in the dashboard in the driver’s view. The display 
enables us to show in the car: onboard flagging (green, CODE60 etc.), messages from Race Control to 
individual or all cars, automated time penalty serving system with in-car countdown and data made 
available to competitors by Timeservice.

RGB LIGHT

The RGB Light need to be placed next to the LUMIRANK Display on the front wind shield. It shows when teams are allowed to change the tires of the car.

- Datalogger

  - Classes GT3 & GT4 must be equipped with a datalogger from AIM Scrutineering.

  - Classes GTX & TCX: only selected cars must be equipped with a datalogger from AIM Scrutineering.

- Accident Data Recorder (GT3 only)
All cars in class GT3 must be equipped with an Accident Data Recorder according to the FIA Technical Regulations for GT3 cars: click here for the FIA Technical List with information on the ADR

- Cosworth Datalogger (992 only)

Class 992 must be equipped with a Porsche data logger according to the Porsche parts catalogue.

- 2 USB Sticks (992 only)
Teams must use 2x 9F0054911A (Speicher RLU Rugged USB/Memory RLU Rugged USB), labelled with the start number of the car. Please note that according to art. 22.1 of the Sporting Regulations, both USB devices must be presented during scrutineering.

Alternatively, Teams may also use Genioma kit with part number G-RLU-A, with 2 USB storage devices labelled with the start number of the Car. The USB storage devices must be formatted by the Team as RLU device using the Pi Cosworth Toolset. Please note that, according to art. 22.1 of the Sporting Regulations, both USB devices must be presented and functional during scrutineering.

- RFID Tag

The RFID Tag is provided by Creventic and is included in the entry for the race.

- Illuminated backpanels
The illuminated backpanels can be purchased at Creventic. Price € 300 (only required when the race takes place in the dark).